DATE:	July 30, 2026

 FROM:         Intel Applications Support - Oregon

SUBJECT:	Standard Advanced/ML  BIOS Release7.0

About This Release

o	BIOS 1.00.07.DB0_ Release 7.0

Features/Errata Fixed In This Release

Release 7.0

- Closed window between CMOS write and CMOS checksum to reduce possibility of a power down during POST causing a CMOS checksum error.

- Added additional checks for displaying the DMI VPD information during  POST.  This BIOS will not display a serial number or product name unless valid data is present.

- Relaxed DRAM Timings in ECC mode.

- Modified PCI Latency Timer values.

- Updated static DMI information

Release 6.0

- Changes for WIN95ä supplement LOGO acceptance tests.

- Added code to prevent ECC errors while memory initializing.

- Fixed issue w/ parity errors.

- Return real status after disable/enable Cache.

- Included monitoring IDE IRQs for system inactivity events.

- Updated DMI structures.

- Move IRQ 12 in IRQ priority allocation list so mouse IRQ  not given away prematurely.

- Added ability to display the DMI product type and serial number during POST and the DMI interface to enable/disable it.

- Modified the Latency Timer code to allow an intelligent method for determining the actual value to be programmed into each  PCI device's timer register.

- Corrected situation where ESCD could not be updated because flash block was left in an indeterminate state by an erase abort (power interruption during an erase operation).

- IDE Translation Mode updated so that only 'Standard CHS' and 'Extended CHS' are offered if the 'IDE Device Configuration' option is set to 'User Definable'.

- Corrected issue with hard drive configuration in the Setup utility.  Under certain system configurations, the three configuration entries on a hard drive screen (IDE Translation,  Multiple Sector, Fast PIO) could potentially alter the configuration for a drive defined on a different hard drive screen.

- Changes Serial Port 2 IR mode to half-duplex.

- Fixed issue where motherboard function 0 data was nulled if no mouse installed.

- Fixed display of IR informative node. Previously always displayed as disabled.

- Shadow FFhs from E0000h to E7FFFh, if possible, so EMM programs can use area for UMBs. Also for Microsoft WIN95ä logo purposes.

- Fixed issue  where PCI SIG card failed.

- Fixed the long time to boot issue when certain ISA video cards where used.

- Move POST scratch buffer above 1MB so add-in cards that search for free memory can find some.

- This code corrects a issue  where we were not handling PnP calls properly in 16 bit protected mode.

- Fixed issue of system hang during ESCD clear using third party's ‘Clear ESCD' program.

- Resolves the situation where multiple ESCD Slots were being shown for a single Multi-function device.

- Always insure one interrupt preserved for PCI.

- Fixed issue  where backing out a failed set of resources during auto-configuration didn't clear out the port base address.

- Changed code to prevent device node that used multiple IRQs or multiple DMAs from assigning the same IRQ or DMA to two different resources.

- Provide hook for broken PnP cards when assigning I/O.

- Fixed issue  where PnP cards could get memory from E0000 to FFFFF.

- CPU frequency is detected earlier in POST to satisfy DMI requirements.

- Fixed potential issue of system hanging when no boot device found.

- Added more boundary checking to INT 13.

- Fixed issue of system hanging when in virtual mode and a jump to the restart vector is executed.

Release 5.0

- Fixed power light to blink during APM power down.

- Changed the Memory Address Drive Strength setting in DRAMEC register of the TXC from 8mA to 12mA.

Release 4.0

o	Included both Static and Dynamic mouse device nodes. The Dynamic node is active

only if the PnP setup option “Win’95ä” is selected, in which case the static device node is removed.  The Dynamic mouse device can now be disabled correctly under Windows 95ä.

o	Added APM 1.2 support

o	Added secure flash implementation.

o	Added new algorithm to detect processor speed

o	Changed PIIX3 MIRQ0 programming to eliminate errata with sightings on secondary channel with CD-ROM devices.

o	Added setup flexibility for Memory Error correction. (If ECC or parity SIMMS are detected then relevant options show up in setup.)

Release 3.0

o	Memory detection changes.

o	FP timings change X-3-3-3.

Release 2.0

o	Setup changes date being programmed incorrectly.